# Setting up a Temperature/Humidy Sensor in your Pico

Overview

In this exercise, we are going to learn how to read temperature and humidity data from the DHT11 temperature/humidity sensor.

Exercise 1: The DHT11

Take out the following items:

- The DHT11

- 2 short wires

- 1 long wire

A DHT11 looks like this:

The DHT 11 has 4 pins.  Look for the side of the DHT11 that is hatched.  From left to right, the pins are:

1: Power pin- must be connected to a power source

2: Data pin- to be connected to a GPIO pin

3- Doesn’t do anything

4- Ground pin- must be connected to ground.

We are going to assemble these as per the following diagram:

I found it easiest to connect the DHT11 by flipping it so the power pin is closest to the Pico.  Put the DHT11 on the set of pins closest to the center divider.

The DHT11 can be read at most once per second.  It produces two outputs:

- A temperature output in Celsius

- A humidity output as a humidity percentage with 0 being completely dry and 100 being water should be condensing

To use the DHT11, we first invoke the measure() function which measures the temperature and humidity.  We then extract the temperature and humidity values.

Exercise 2: Our Code

In a new sketch, type the following:

‬‬‬‬

There is one unusual variation in a command, and one new command.  The variation in command is in setting the input pin as ADC.  ADC stands for analog to digital conversion.  We are reading analog input and converting into a form a computer chip can understand.

The ADC library also doesn’t use Pin.value(), instead using ADC.read_16().  Pin.value() returns a value between 0 and 1.  ADC.read_u16 converts the infinite variation of an analog signal into 216=65536 possible values

Exercise 3: Monitoring the sensor

If your sensor is working, you should see output like this:

If you breathe on the sensor, the temperature should rise.  If you hold the sensor, the humidity level should rise.

Take a screenshot of the humidity sensor output changing and upload it to Canvas.